# Smart Agricultural Monitoring: Leveraging Machine Learning for Precision Disease Detection and Crop Health Optimization

Prasad Pujar
Department of Computer Science and Engineering
KLS Gogte Institute of Technology
Belagavi 590008, India
pmpujar@git.edu

Shubham Godse  
Department of Computer Science and Engineering
KLS Gogte Institute of Technology
Belagavi 590008, India
shubhamgodseofficial@gmail.com

Irfan Hussain Lone 
Department of Computer Science and Engineering
KLS Gogte Institute of Technology
Belagavi 590008, India
irfanmurtaza7051@gmail.com

Abhishek Shelke 
Department of Computer Science and Engineering
KLS Gogte Institute of Technology
Belagavi 590008, India
abhishekshelke421@gmail.com

Rohan O Chikorde
Department of Computer Science and Engineering
KLS Gogte Institute of Technology
Belagavi 590008, India
rohanchikorde36@gmail.com

Abstract— In the face of rising global concerns about food security and agricultural productivity, particularly with the challenges posed by climate change and pest outbreaks, there is a growing need for efficient and smart farming techniques. This research aims to develop a Smart Disease Detection and Field Monitoring System using IoT (Internet of Things) sensors, coupled with AI/ML (Artificial Intelligence and Machine Learning) algorithms, to enhance plant health management and optimize field monitoring. The system integrates multiple sensors for temperature, humidity, and soil moisture, which are crucial for determining crop health. Additionally, AI-based disease detection models are employed to recognize plant diseases from images of leaves and other plant parts. The core components of this system include the use of an ESP8266 microcontroller that connects various sensors to the internet, enabling real-time data transmission to a Firebase cloud database. The collected data is then used to train an AI/ML model, which helps identify potential diseases and anomalies in the plants' health. This model processes visual data from cameras, analyzes the environmental factors such as temperature, humidity, and soil moisture, and provides actionable insights into plant health, guiding the appropriate interventions such as irrigation or pesticide application. The disease detection algorithm employed utilizes a Convolutional Neural Network (CNN), which has been trained to recognize patterns associated with specific plant diseases, ensuring accurate and early detection. The system also incorporates a dashboard interface, built using Flask or Django, where farmers and users can monitor real-time data from the field, track historical trends, and receive recommendations for managing plant health.

Keywords—Agricultural Productivity, Climate Change, Pest Outbreaks, Smart Farming, IoT Sensors, Artificial Intelligence (AI), Machine Learning (ML), Plant Health Management, Field Monitoring, ESP8266 Microcontroller, Real-time Data Transmission, Convolutional Neural Network (CNN) Visual Data Analysis, Irrigation Management, Pesticide Application, Crop Health Optimization, Plant Disease Recognition.

# Introduction

Agriculture is a vital sector that sustains life and supports economies worldwide. It plays an indispensable role in ensuring food security, generating employment, and contributing to the GDP of many nations. However, the agricultural industry faces numerous challenges, including declining crop yields, increasing pest and disease outbreaks, and resource mismanagement [1]. Among these, plant diseases remain one of the most significant threats, responsible for substantial yield losses and economic setbacks globally. These issues are further compounded by the unpredictable effects of climate change, which exacerbate the spread and intensity of diseases, making traditional agricultural practices less effective [2]. Farmers, particularly those in small and medium-scale operations, struggle to identify diseases promptly, leading to delayed intervention, reduced productivity, and, in some cases, total crop failure. The need for innovative, data-driven solutions to tackle these issues has never been more urgent. In this context, the convergence of modern technologies, such as the Internet of Things (IoT), Artificial Intelligence (AI), and cloud computing, has opened new possibilities for transforming traditional farming into a more efficient, intelligent, and sustainable practice [3]. IoT enables real-time monitoring of critical environmental parameters like temperature, humidity, and soil moisture, providing continuous insights into field conditions. AI, on the other hand, brings precision and efficiency by leveraging machine learning algorithms to analyze plant images and accurately detect diseases at an early stage [4]. Combined, these technologies provide a holistic approach to addressing the dual challenges of disease detection and field monitoring. The proposed Smart Disease Detection and Field Monitoring System seeks to integrate IoT based environmental sensing with AI-driven disease diagnosis to empower farmers with real time, actionable insights. By deploying sensors in the field, the system continuously monitors key environmental conditions, ensuring that critical thresholds are identified and communicated promptly. Simultaneously, a machine learning-based disease detection module analyzes images of affected crops to identify diseases with high accuracy, enabling early intervention and minimizing losses [4]

# Objectives

## Real-Time Monitoring: Develop a system to continuously monitor environmental parameters such as temperature, humidity, and soil moisture using IoT sensors.

## Early Disease Detection: Implement a machine learning model to accurately identify plant diseases from images, enabling early intervention and reducing crop loss.

## Automated Irrigation: Integrate a soil moisture sensor with an automated water pump system to optimize irrigation and conserve water resources.

## User-Friendly Interface: Create an intuitive web or mobile dashboard for real-time data visualization, disease alerts, and system control.

## Cloud-Based Data Management: Utilize a cloud platform like Firebase for real-time data storage, retrieval, and synchronization across devices.

## Affordable and Scalable Solution: Design a cost-effective system that can be scaled for small to medium-sized farms, making it accessible to a wide range of users.

## Sustainability and Efficiency: Promote resource efficiency and sustainable farming practices through data-driven decision-making.

## Integration and Adaptability: Ensure the system can be adapted for various crops, regions, and environmental conditions.

# METHODOLOGY

## Identifying Key Challenges in Agriculture through Surveys and Interviews

To design an effective and farmer-centric Smart Plant Disease Detection and Field Monitoring System, it is crucial to understand the specific challenges faced by farmers. This study employed surveys and interviews to gather insights directly from the primary stakeholders—farmers. These consultations focused on identifying common plant diseases, critical environmental parameters affecting crop health, and operational issues such as irrigation management.

By aligning the system's features with the actual needs of farmers, this research aims to create a practical, scalable, and affordable solution that can drive sustainable agricultural practices.

## Key Challenges Identified through Surveys and Interviews

### Common Plant Diseases in the Region

Farmers were asked about their experiences with plant diseases, including their frequency, impact, and existing methods of management.

Findings

- Diseases like powdery mildew, bacterial blight, and late blight were commonly reported in crops like wheat, rice, and vegetables.

- Existing detection methods (manual observation) were time-consuming and often inaccurate, leading to delayed intervention.

- Farmers expressed the need for a reliable, early-warning system to reduce crop losses.

### Environmental Parameters Critical for Crop Health

Farmers emphasized the importance of monitoring specific environmental conditions to ensure optimal crop growth and prevent diseases [5].

Findings

- Temperature: Significant fluctuations were linked to disease outbreaks and poor crop performance.

- Humidity: High humidity was identified as a trigger for fungal infections.

- Soil Moisture: Variability in soil moisture led to over-irrigation or under-irrigation, affecting root health and yield.

### Challenges in Irrigation and Water Management

Efficient water management emerged as a critical issue for many farmers, particularly in water-scarce regions [6]

Findings

- Lack of real-time soil moisture data led to overuse of water, increasing operational costs and reducing water availability.

- Farmers expressed interest in an automated system to optimize irrigation schedules based on soil and weather conditions

## Defining Functional and Non-Functional Requirements

These specify the primary functions the system must perform to address the farmers’ challenges effectively

### Functional Requirements

Real-Time Monitoring and Control:

- Continuous monitoring of temperature, humidity, and soil moisture using IoT sensors.

- Automated irrigation control based on real-time soil moisture levels.

Disease Detection and Alerts:

- Use of AI-powered image recognition to classify plant diseases with high accuracy.

- Real-time notifications to farmers with suggested remedies for detected diseases.

Data Visualization:

- A user-friendly dashboard displaying environmental conditions and disease predictions.

- Historical data storage for trend analysis and decision-making.

### Non-Functional Requirements

These define the system's overall characteristics to ensure reliability, scalability, and user satisfaction:

Scalability:

- System design should support both small-scale farms and large commercial operations.

- Modular architecture to allow the addition of new sensors or features.

Affordability:

- Components like sensors and microcontrollers should be cost-effective to make the system accessible to small-scale farmers.

- Free or low-cost web and mobile applications for system interaction.

Robustness and Reliability:

- Accurate sensor readings even in harsh weather conditions.

- Reliable AI model performance with minimal false positives or negatives.

Energy Efficiency:

Use of energy-efficient sensors and microcontrollers to minimize operational costs.

## Integration of Findings into System Design

The insights gathered through surveys and interviews directly influenced the design of the Smart Plant Disease Detection and Field Monitoring System [7]. For example:

- The inclusion of IoT sensors for temperature, humidity, and soil moisture was guided by the importance of these parameters as reported by farmers.

- The AI model for disease detection was trained on datasets containing images of diseases most frequently encountered in the region.

- The automated irrigation system was developed to address water management issues highlighted during interviews.

# SYSTEM ANALYSIS

The Smart Disease Detection and Field Monitoring System is designed to address critical challenges in agriculture, such as early detection of plant diseases, efficient water management, and real-time monitoring of environmental parameters. The system's analysis is structured into problem identification, system requirements, and feasibility study [8].

## Problem Identification

Agricultural productivity is often hampered by:

- Plant Diseases: Delayed or inaccurate identification of plant diseases leads to reduced yields and economic losses.

- Inefficient Irrigation: Over-irrigation or under-irrigation due to lack of real-time data results in poor crop health and resource wastage.

- Manual Efforts: Traditional methods of field monitoring are labor-intensive, time consuming, and error-prone.

- Data Accessibility: Farmers lack actionable insights due to limited access to real-time and historical environmental data.

## SYSTEM REQUIREMENTS

### Software Requirements

The software components of the Smart Disease Detection and Field Monitoring System include tools and platforms for data collection, processing, visualization, and machine learning integration. Below is a brief overview

- Programming and Development Tools

The project utilizes Arduino IDE for programming and uploading firmware to the ESP8266 microcontroller, enabling sensor integration and data communication. Python is employed for developing the machine learning model and handling backend processing, ensuring accurate plant disease detection and efficient system performance. Additionally, Flask serves as the framework for creating an interactive web or mobile dashboard, facilitating user interaction through real-time data visualization and control features.

- Cloud Platform

Firebase is used as the cloud platform for the project, featuring a Realtime Database to store and synchronize sensor data and user actions efficiently. Firebase Hosting ensures seamless deployment of the web application, enabling real-time accessibility. Additionally, Firebase Authentication provides secure user login and access control, safeguarding data and maintaining system integrity.

- Machine Learning Framework

The project employs TensorFlow or PyTorch to train and deploy a Convolutional Neural Network (CNN) model for accurate plant disease detection. For integration with IoT platforms or web interfaces, the model is optimized using TensorFlow Lite, ensuring efficient and lightweight deployment suitable for resource-constrained environments.

### Hardware Requirements

DHT11 Sensors: Measure temperature and humidity.

Soil Moisture Sensors: Assess soil water content for irrigation management.

ESP8266 Microcontroller: Processes data and communicates with the cloud platform

Fig.1 Soil Moisture Sensor

Fig. 2 Humidity Sensor

Fig. 3 Motor Pump

Fig. 4 NodeMCU

# SYSTEM DESIGNING

## System Architecture Overview

Fig. 5 System Circuit Diagram

## Disease Detection Using AI/ML

The disease detection workflow involves several stages, from image acquisition to classification and output generation The disease detection component of the Smart Plant Disease Detection and Field Monitoring System is based on a Machine Learning (ML) algorithm designed to accurately classify plant diseases using image data. This algorithm is central to the system, allowing farmers to detect diseases at early stages, reducing the need for manual inspections, and providing actionable insights in real-time. The primary technique used for disease detection is Convolutional Neural Networks (CNNs), a deep learning method particularly suited for image classification tasks.[9]

Fig. 6 Disease Detection Working

Image Acquisition: The system allows the farmer to upload images of plant leaves via a web or mobile interface. These images can be captured using smartphones or digital cameras. The quality and clarity of the image are crucial for accurate disease detection. The image is pre-processed (if necessary) to improve quality before being fed to the model.

Pre-Processing: Image Normalization: To ensure that all images are treated equally, the input images are normalized (resized, color normalized) so that they fit the expected dimensions for the model.

Data Augmentation: To increase the model's robustness and prevent overfitting, techniques such as rotation, flipping, cropping, and zooming are applied to the images.

Feature Extraction (CNN-based):

Convolutional Layers: The core of the disease detection model is a CNN. These networks are designed to automatically detect and learn relevant features in an image, such as edges, textures, and shapes, without the need for manual feature engineering.

Activation Functions: After each convolution operation, an activation function (e.g., Re LU) is used to introduce non-linearity, enabling the network to learn more complex patterns.

Model Training: A CNN model is trained using a large dataset of plant images, labeled with disease categories (e.g., healthy, leaf blight, rust, powdery mildew). The dataset should be representative of various diseases that affect the crops the farmer is interested in monitoring.

bDuring training, the model adjusts the weights of the convolutional layers to minimize the error between the predicted and actual disease labels.

Classification: Once the model has been trained, it can classify new, unseen images by passing them through the layers of the CNN. The network predicts the disease or status of the plant (healthy or diseased) based on learned features. The output is typically a probability distribution over the classes (diseases), and the class with the highest probability is chosen as the prediction.

Post-Processing: Disease Confidence: The system displays the disease prediction with the confidence score, indicating how confident the model is in its classification. If the confidence score is low, the system may request a re-check or more data to improve prediction accuracy.

Visualization: The results, including the disease name and a confidence score, are shown on the web or mobile interface for easy interpretation by the farmer.

# Disease detection processing algorithm

## Convolutional Neural Network (CNN)

The Convolutional Neural Network (CNN) model is a key component of the Smart Plant Disease Detection and Field Monitoring System. It processes crop images to accurately identify diseases based on visual symptoms [10]. The working of the CNN model is detailed below:

Data Collection and Preprocessing

- Data Collection: A large dataset of annotated crop images, including healthy and diseased samples, is used for training the model.

- Preprocessing: The images are resized, normalized, and augmented to enhance model performance and prevent overfitting. Techniques like rotation, flipping, and brightness adjustments are applied to simulate real-world conditions.

Model Architecture

- Convolutional Layers: Extract features such as patterns, textures, and edges from input images. These layers use filters to identify disease-specific features.

- Pooling Layers: Reduce the dimensionality of feature maps while retaining key information, optimizing computational efficiency.

- Fully Connected Layers: Integrate extracted features and classify them into predefined categories (e.g., healthy or specific diseases).

- Activation Functions: Functions like ReLU (Rectified Linear Unit) introduce non-linearity, helping the model learn complex patterns.

Fig.7 CNN Architecture

Training the Model

The model is trained using the annotated dataset, employing supervised learning techniques.

- Loss Function: Cross-entropy loss evaluates the model's predictions against the true labels, guiding the optimization process.

- Optimizer: Algorithms like Adam adjust model weights to minimize the loss function.

- Validation: A portion of the dataset is reserved for validation to monitor model performance and prevent overfitting.

Deployment

Once trained, the model is converted to a lightweight format using TensorFlow Lite, ensuring compatibility with IoT platforms and web interfaces.

The model is integrated into the system, allowing farmers to upload crop images through a web or mobile application.

Inference and Output

- Image Input: The user uploads a crop image, which is preprocessed before being passed to the CNN model.

- Prediction: The model analyzes the image and classifies it as healthy or diseased, specifying the disease type if applicable.

- Output: The system provides the diagnosis along with recommended actions or treatments.

This CNN-based approach enables quick, accurate, and automated plant disease detection, empowering farmers to take timely action and minimize crop losses.

## Data Preprocessing and Augmentation

## Normalization: Images are resized to a standard size (e.g., 224x224 pixels) to ensure uniform input dimensions for the CNN.

Data Augmentation: This helps the model generalize better by artificially increasing the size of training dataset.

Random transformation like rotation, scaling, flipping, and zooming are applied to training images.

# IMPLEMENTATION

## Integration of AI/ML and IoT for Smart Agriculture

This project combines Artificial Intelligence/Machine Learning (AI/ML) and Internet of Things (IoT) technologies to develop a Smart Plant Disease Detection and Field Monitoring System. Leveraging IoT sensors such as soil moisture, temperature, and humidity sensors, the system monitors environmental conditions critical for crop health. Data is collected via the ESP8266 microcontroller and synchronized in real-time to Firebase, a cloud platform, for seamless storage and retrieval. The integration of a web interface facilitates real-time visualization and control, allowing users to monitor field conditions and automate irrigation systems based on sensor data [11].

## AI/ML-Based Disease Detection

The disease detection system employs Convolutional Neural Networks (CNNs) to classify plant images into healthy or diseased categories, such as Leaf Blight, Powdery Mildew, or Rust. Training involves preprocessing a labeled dataset with consistent image sizes (224x224 pixels) and applying data augmentation techniques like rotation and flipping to improve model robustness. The CNN architecture includes multiple convolutional, pooling, and fully connected layers, utilizing ReLU and SoftMax activations for feature extraction and classification. The model is trained using Python libraries such as TensorFlow and Keras and optimized with the Adam optimizer and cross-entropy loss. Performance is evaluated with metrics like accuracy, precision, recall, and F1-score, ensuring reliable predictions [12].

## IoT-Based Yield Monitoring

The yield monitoring system integrates sensors to measure environmental conditions and optimize irrigation. Soil moisture, temperature, and humidity data are captured by sensors connected to the ESP8266 microcontroller, which transmits the readings to Firebase. The system automates irrigation through a relay-controlled water pump, ensuring optimal water usage based on soil moisture levels. Real-time updates and historical data trends are displayed on a user-friendly web interface built with Flask or Django. The Firebase platform ensures reliable communication and storage, enabling scalability and real-time control [13].

## System Deployment and Integration

The AI/ML model for disease detection is deployed using TensorFlow Lite for efficient inference on IoT or web platforms. Users can upload plant images through the web interface, where the trained model processes the image to predict health status or specific diseases. Simultaneously, sensor data from the field is visualized using graphs and gauges, with alerts generated for critical conditions. The system's scalability and affordability make it suitable for small-scale farmers, while its intuitive interface ensures ease of use for users with minimal technical expertise.

# Result of Project Development and Deployment

The development and deployment of the Smart Plant Disease Detection and Field Monitoring System have yielded promising results, demonstrating its practical utility in agricultural management. The Convolutional Neural Network (CNN) model achieved high accuracy in identifying plant diseases from uploaded images, classifying them into categories such as "Healthy," "Leaf Blight," and "Powdery Mildew." With consistent preprocessing and robust training using augmented datasets, the model generalizes well, offering reliable disease predictions. This ensures timely diagnosis and minimizes crop losses.

The IoT-based monitoring system successfully integrates sensors for real-time data collection of environmental parameters such as soil moisture, temperature, and humidity. The ESP8266 microcontroller effectively transmits sensor data to Firebase, which synchronizes the information and displays it on an interactive web dashboard. This enables farmers to monitor field conditions remotely and automate irrigation based on soil moisture levels, optimizing water usage and reducing manual effort. Alerts for critical conditions provide additional support in maintaining crop health.

Deployment on a web interface ensures seamless interaction between the user and the system. The lightweight CNN model, optimized with TensorFlow Lite, facilitates efficient disease detection, while the Firebase-backed dashboard provides a comprehensive view of field conditions. The system's scalability and affordability make it an accessible solution for both small and large-scale farmers, aligning with the goal of promoting sustainable and technology-driven agriculture.

# APPLICATIONS AND SCOPE OF PROJECT

## Early Disease Detection and Management

The system provides timely identification of plant diseases through AI-powered image analysis, allowing farmers to take corrective actions before the diseases spread. This reduces crop loss and ensures better yield quality, significantly impacting food security and agricultural productivity.

## Precision Agriculture

By leveraging real-time environmental data from sensors, the system supports precision agriculture practices. Farmers can optimize irrigation, manage resources efficiently, and maintain ideal growing conditions for crops, leading to higher productivity and resource conservation.

## Water Resource Management

The integration of soil moisture sensors and automated irrigation systems enables efficient water usage. This is particularly beneficial in regions facing water scarcity, as the system ensures water is used only when needed, preventing wastage and reducing costs.

## Smart Farming for Small-Scale Farmers

Designed with affordability and scalability in mind, the system empowers small-scale farmers to adopt smart farming practices. With its intuitive web interface and automated processes, it lowers the technological barriers, making advanced agricultural management accessible to everyone.

## Real-Time Field Monitoring

The IoT-enabled monitoring system allows farmers to track critical environmental parameters such as temperature, humidity, and soil moisture in real time. The interactive dashboard provides actionable insights and alerts, enabling farmers to address potential issues promptly.

## Sustainable Agriculture Practices

The system promotes sustainability by minimizing resource wastage, reducing reliance on chemical interventions, and supporting environmentally friendly farming practices. By optimizing inputs like water and fertilizers, it helps in maintaining ecological balance while enhancing crop yields.

# CONCLUSION

The Smart Plant Disease Detection and Field Monitoring System successfully integrates AI/ML and IoT technologies to address critical challenges in modern agriculture. By combining advanced Convolutional Neural Networks (CNNs) for disease detection with real-time environmental monitoring, the system provides farmers with actionable insights, enabling timely intervention and better management of crops. The project demonstrates the potential of technology to reduce crop losses, improve productivity, and optimize resource utilization, especially water, through automated irrigation.

The AI/ML model, trained on diverse datasets, achieves high accuracy in classifying plant health and detecting diseases. The use of TensorFlow Lite ensures efficient deployment on IoT platforms, facilitating real-time inference even in low-resource environments. The IoT system, powered by the ESP8266 microcontroller and Firebase cloud platform, captures and transmits data seamlessly, providing farmers with real-time updates and historical trends. This integration creates a user-friendly and scalable solution, catering to farms of varying sizes.

In addition to its technical capabilities, the system is designed to be cost-effective and intuitive, making it accessible to small-scale farmers with minimal technical expertise. The web interface, equipped with interactive dashboards and alert systems, empowers users to take informed decisions and respond proactively to environmental or crop health issues. This ensures not only improved yields but also long-term sustainability in farming practices.

In conclusion, this project serves as a robust example of how AI and IoT can transform agriculture by addressing key challenges like disease management and resource optimization. By leveraging cutting-edge technology, the system supports farmers in achieving higher efficiency and productivity while contributing to sustainable agricultural development. Future enhancements could focus on expanding the system’s capabilities to include predictive analytics and support for a wider range of crops and diseases, further broadening its impact.

The project utilizes Arduino IDE for programming and uploading firmware to the ESP8266 microcontroller, enabling sensor integration and data communication. Python is employed for developing the machine learning model and handling backend processing, ensuring accurate plant disease detection and efficient system performance. Additionally, Flask serves as the framework for creating an interactive web or mobile dashboard, facilitating user interaction through real-time data visualization and control features .measurement and others are deliberate, using specifications that anticipate your paper as one part of the entire proceedings, and not as an independent document. Please do not revise any of the current designations.

# Conclusion

S Selvan, Shilpa & Wahid, Aseeya & Patel, Abhishek & Kumar, Vijay & Sahu, Parmanand. (2021). Challenges in Indian Agriculture. Agricultural Reviews. 10.18805/ag.R-2103.

Yadav, Dipak & Gupta, Aman & Gupta, Priyanka & Ranjan, Supriya & Gupta, Vishal & Badhai, Sirpat. (2020). Effects of Climate Change on Agriculture.

Vadapalli, Adithya & Peravali, Swapna & Dadi, Venkatarao. (2020). "Smart Agriculture System using IoT Technology" Publisher: International Journal of Advance Research in Science and Engineering (2319-8354). 09. 58-65.

Tripathi, Padmesh & Kumar, Nitendra & Rai, Mritunjay & Shukla, Pushpendra & Verma, Kailash. (2023). Applications of Machine Learning in Agriculture. 10.4018/978-1-6684-6418-2.ch006.

Raza, Muhammad Haseeb et al. “Environmental and Health Impacts of Crop Residue Burning: Scope of Sustainable Crop Residue Management Practices.” International journal of environmental research and public health vol. 19,8 4753. 14 Apr. 2022, doi:10.3390/ijerph19084753

Basukala, Amit & Eschenbach, Annette & Rasche, Livia. (2024). Effect of irrigation canal conveyance efficiency enhancement on crop productivity under climate change in Nepal. Environmental Monitoring and Assessment. 196. 10.1007/s10661-024-13405-4.

Judijanto, Loso & Vandika, Arnes & Priyana, Yana. (2024). Implementation of Internet of Things (IoT) in Information System. West Science Information System and Technology. 2. 269-279. 10.58812/wsist.v2i02.1212.

An Overview of the Research on Plant Leaves Disease detectionusing Image Processing Techniques”, January 2014, Ms. Kiran R.Gavhale.

Apple Scab and Marsonina Coronaria Diseases Detection in Apple Leaves Using Machine Learning January 2018 authors Swati Singh, Sheifali Gupta, International Journal of Pure and Applied Mathematics

O'Shea, Keiron & Nash, Ryan. (2015). An Introduction to Convolutional Neural Networks. ArXiv e-prints.

Kushagra Sharma, Shiv Kumar Shivandu,Integrating artificial intelligence and Internet of Things (IoT) for enhanced crop monitoring and management in precision agriculture,Sensors International, Volume 5,2024,100292,ISSN 2666-3511

Plant Leaf Disease Detection using Inception Resnet V2 -CNN Vishal Tanawade, Sarthak Thorat, Suraj Shirude, Hitesh Saswadkar, Pratik Karhekar, Prof. V. V. Waykule

IoT based temperature and humidity monitoring framework Rafizah Ab Rahman, Ummi Raba'ah Hashim, Sabrina Ahmad

- IEEE conference templates contain guidance text for composing and formatting conference papers. Please ensure that all template text is removed from your conference paper prior to submission to the conference. Failure to remove template text from your paper may result in your paper not being published.